Supporting Information

Techno-economic Analysis of Using Ammonia as an Energy Carrier for Renewable Energy Conversion and Storage

Yun Liua,1, Mingyang Luoa,1, Wenzhi Lia, Chuncheng Zhoua, Xiaoyu Huoa, Zhefei Pan b,c*, Rong Chenb,c*, Liang Ana,*

a Department of Mechanical Engineering, The Hong Kong Polytechnic University, Hung Hom, Kowloon, Hong Kong SAR, China

b Key Laboratory of Low-grade Energy Utilization Technologies and Systems (Chongqing University), Ministry of Education of China, Chongqing University, Chongqing, 400044, China

c Institute of Engineering Thermophysics, School of Energy and Power Engineering, Chongqing University, Chongqing, 400044, China

1 Equal contribution.

*Corresponding author.

Email: zhefei.pan@cqu.edu.cn (Zhefei Pan)

Email: rchen@cqu.edu.cn (Rong Chen)

Email: liang.an@polyu.edu.hk (Liang An)

# Energy requirements of terminal application.

In this study, considering the typical mature applications of ammonia-based power generation technologies, the terminal application is assumed to be a ship, which power requirements are listed in Table S1. Taking the fuel consumption of maritime transport, in accordance with its running time and distance, it can be calculated that the annual energy consumption is equal to the 2710.3 MWh. The same amount of energy can be supplied via the electricity generated by the internal combustion engine or fuel cells. Using ammonia as an energy carrier for renewable energy conversion and storage via electricity-ammonia-electricity generally includes several stages, including renewable electricity generation [1-11], green hydrogen production [12-18], green ammonia synthesis [19, 20], ammonia storage and transport [21], ammonia cracker, and electricity generation by internal combustion engine [22-27] and fuel cells [25, 28-34]. The techno-economic parameters used for analyzing various stages are listed in Table S2, which are referred from public data sources such as reports, research papers, and websites etc.

Table S1. Terminal application electricity requirements.

| Parameters | Value |
| --- | --- |
| Total travelling time (hours) (1 September 2020 - 31 May 2022) | 6604 |
| Average fuel consumption (Litres/hour) | 34.20 |
| Average fuel cost (HK$/hour) | 711.00 |
| Average total operating cost (HK$/hour) | 874.00 |
| Annual fuel consumption (Litre) | 82129.75 |
| Annual energy consumption (MWh) | 2710.30 |
| Main generator sets (two sets) | 275 kW @1800 rpm, 60 Hz |
| Auxiliary generator sets (two sets) | - W @1500 rpm |

Table S2. The reference of the techno-economic parameters used in this study.

| Years | 2023 | 2030 | 2040 |  | 2050 |  |  | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Renewable electricity generation |  |  |  |  |  |  |  |  |
| Solar cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 |  | 440.00 |  | 363.00 |  | [8-10] |
| Solar cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 |  | 374.00 |  | 308.60 |  | [8-10] |
| Solar cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 |  | 308.60 |  | 253.00 |  | [8-10] |
| Solar power plants O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % |  | 2 % |  | 2 % |  | [8-10] |
| Wind cost (USD/kW) | 1250.00 | 913.00-1127.50 |  | 788.00-1070.30 |  | 638.00-1028.50 |  | [11] |
| Wind power plants O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % |  | 2 % |  | 2 % |  | [11] |
| Hydrogen production via PEM electrolyzer |  |  |  |  |  |  |  |  |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 |  | 495.00 |  | 440.00 |  | [14] |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 |  | 495.00 |  | 440.00 |  | [14] |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % |  | 5 % |  | 5 % |  | [14] |
| Hydrogen production via alkaline electrolyzer |  |  |  |  |  |  |  |  |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 |  | 385.00 |  |  | [14] |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 |  | 275.00 |  |  | [14] |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % |  | 5 % |  |  | [14] |
| Ammonia production |  |  |  |  |  |  |  |  |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 |  | 0.99 |  |  | [19, 20] |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % |  | 5 % |  |  | [19, 20] |
| Ammonia transport |  |  |  |  |  |  |  |  |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 |  | 0.02225 |  |  | [21] |
| O & M cost (USD/t) (of the CAPEX cost) | 2 % | 2 % | 2 % |  | 2 % |  |  | [21] |
| Ammonia storage |  |  |  |  |  |  |  |  |
| Cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 |  | 0.00379 |  | [21] |  |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % |  | 2 % |  | [21] |  |
| Electricity generation |  |  |  |  |  |  |  |  |
| ICE cost (USD/kW) | 521.00 | 425.00 | 288.00 |  | 151.00 |  | [24, 27] |  |
| ICE O & M cost (USD/kW) (of the CAPEX cost) | 4 % | 4 % | 4 % |  | 4 % |  | [24, 27] |  |
| SOFC cost (USD/kW) | 2200.00 | 1617.00 | 1067.00 |  | 800.00 |  | [34] |  |
| SOFC O & M cost (USD/kW) (of the CAPEX cost) | 5.5 % | 5.5 % | 5.5 % |  | 5.5 % |  | [34] |  |
| AEMFC cost (USD/kW) | 2211.90 | 1308.80 | 403.60 |  | 141.80 |  | [28, 29] |  |
| AEMFC O & M cost (USD/kW) (of the CAPEX cost) | 5.5 % | 5.5 % | 5.5 % |  | 5.5 % |  | [28, 29] |  |
| PEMFC cost (USD/kW) | 166.00 | 80.00 | 70.00 |  | 60.00 |  | [25, 32] |  |
| PEMFC O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % |  | 5 % |  | [25, 32] |  |

# The techno-economic parameters of ICE-based electricity-ammonia-electricity route.

Table S3. The techno-economic parameters of ICE application route from 2023 to 2050.

| Techno-economic parameters | Data |  |  |  |
| --- | --- | --- | --- | --- |
| ICE | 2023 | 2030 | 2040 | 2050 |
| Energy output per year (MWh) | 2710.30 | 2710.30 | 2710.30 | 2710.30 |
| Energy input per year (MWh) | 6775.75 | 6775.75 | 6775.75 | 6775.75 |
| Energy efficiency (%) | 40.00 | 40.00 | 40.00 | 40.00 |
| Power of ICE (kW) | 700.00 | 700.00 | 700.00 | 700.00 |
| Cost (USD/kW) | 521.00 | 425.00 | 288.00 | 151.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 4 % | 4 % | 4 % | 4 % |
| Total cost (Million USD) | 0.80 | 0.65 | 0.44 | 0.23 |
| ICE LCOE (USD/kWh) | 0.00984 | 0.00799 | 0.00541 | 0.00283 |
| Ammonia storage |  |  |  |  |
| Energy output per year (MWh) | 6775.75 | 6775.75 | 6775.75 | 6775.75 |
| Energy input per year (MWh) | 7058.09 | 7058.09 | 7058.09 | 7058.09 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 | 0.00379 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.10 | 0.07 | 0.05 | 0.04 |
| Ammonia storage LCOE (USD/kWh) | 0.00049 | 0.00034 | 0.00025 | 0.00020 |
| Ammonia transport (ship) |  |  |  |  |
| Energy output per year (MWh) | 6775.75 | 6775.75 | 6775.75 | 6775.75 |
| Energy input per year (MWh) | 7058.09 | 7058.09 | 7058.09 | 7058.09 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 | 0.02225 |
| O & M cost (USD/t) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.39 | 0.35 | 0.30 | 0.24 |
| Ammonia transport LCOE (USD/kWh) | 0.00192 | 0.00172 | 0.00148 | 0.00118 |
| Ammonia production, 1.007 MW |  |  |  |  |
| Total input hydrogen (t) | 244.34 | 244.34 | 244.34 | 244.34 |
| Total output ammonia (t) | 1357.35 | 1357.35 | 1357.35 | 1357.35 |
| Total energy output (MWh) | 7058.09 | 7058.09 | 7058.09 | 7058.09 |
| Hydrogen conversion rate (%) | 98 | 98 | 98 | 98 |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 | 0.99 |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 4.71 | 3.88 | 3.05 | 2.49 |
| Ammonia production LCOE (USD/kWh) | 0.02224 | 0.01832 | 0.01440 | 0.01176 |
| Hydrogen production via alkaline electrolyzer, 1.448 MW |  |  |  |  |
| Total energy output (MWh) | 8881.70 | 8881.70 | 8881.70 | 8881.70 |
| Total energy input (MWh) | 13355.97 | 12872.03 | 12250.62 | 11242.66 |
| Energy efficiency (%) | 66.5 | 68-70 | 70-75 | 78-80 |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 | 385.00 |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 | 275.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 2.99 | 2.27 | 1.79 | 1.39 |
| Total cost (Million USD) 100 MW | 2.59 | 1.79 | 1.19 | 1.00 |
| LCOE (USD/kWh) 1 MW | 0.01120 | 0.00851 | 0.00673 | 0.00522 |
| LCOE (USD/kWh) 100 MW | 0.00972 | 0.00673 | 0.00450 | 0.00374 |
| Hydrogen production via PEM electrolyzer, 1.267 MW |  |  |  |  |
| Total energy output (MWh) | 8881.70 | 8881.70 | 8881.70 | 8881.70 |
| Total energy input (MWh) | 12335.70 | 10831.34 | 10573.45 | 10327.56 |
| Energy efficiency (%) | 72 | 82 | 84 | 86 |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.22 | 2.26 | 1.57 | 1.39 |
| Total cost (Million USD) 100 MW | 3.22 | 2.26 | 1.57 | 1.39 |
| Hydrogen production LCOE (USD/kWh) | 0.01208 | 0.00851 | 0.00588 | 0.00522 |
| Renewable electricity generation, Solar 7.623 MW, Wind 5.082 MW |  |  |  |  |
| Total energy output (MWh) | 13355.97 | 13355.97 | 13355.97 | 13355.97 |
| Total energy input (MWh) | 66779.86 | 66779.86 | 66779.86 | 66779.86 |
| Energy efficiency (%) | 20 | 20 | 20 | 20 |
| Solar cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 | 440.00 | 363.00 |
| Solar cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 | 374.00 | 308.60 |
| Solar cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 | 308.00 | 253.00 |
| Wind cost (USD/kW) | 1250.00 | 913.00-1127.50 | 788.00-1070.30 | 638.00-1028.50 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Solar total cost (Million USD) 1 MW | 9.12 | 7.11 | 5.37 | 4.43 |
| Solar total cost (Million USD) 50 MW | 7.75 | 6.04 | 4.56 | 3.76 |
| Solar total cost (Million USD) 100 MW | 6.17 | 4.83 | 3.76 | 3.09 |
| Wind total cost (Million USD) | 10.16 | 7.42-9.17 | 6.41-8.70 | 5.19-8.36 |
| Solar LCOE (USD/kWh) 1 MW | 0.02276 | 0.01774 | 0.01340 | 0.01106 |
| Solar LCOE (USD/kWh) 50 MW | 0.01934 | 0.01507 | 0.01138 | 0.00938 |
| Solar LCOE (USD/kWh) 100 MW | 0.01540 | 0.01205 | 0.00938 | 0.00771 |
| Wind LCOE (USD/kWh) | 0.02536 | 0.01853-0.02288 | 0.01599-0.02172 | 0.01295-0.02087 |
| The lifetime of all equipment is assumed to be 30 years |  |  |  |  |

Table S4. The levelized cost of electricity of different size (1 MW & 100 MW).

| 1 MW | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| ICE LCOE | 0.00984 | 0.00799 | 0.00541 | 0.00283 |
| Solar LCOE 1 MW | 0.02276 | 0.01774 | 0.01340 | 0.01106 |
| Hydrogen production LCOE 1 MW | 0.01120 | 0.00851 | 0.00673 | 0.00522 |
| Ammonia production LCOE | 0.02224 | 0.01832 | 0.01440 | 0.01176 |
| Ammonia transport LCOE | 0.00192 | 0.00172 | 0.00148 | 0.00118 |
| Ammonia storage LCOE | 0.00049 | 0.00034 | 0.00025 | 0.00020 |
| Total LCOE (USD/kWh) | 0.06845 | 0.05462 | 0.04167 | 0.03225 |
| 100 MW | 2023 | 2030 | 2040 | 2050 |
| ICE LCOE | 0.00984 | 0.00799 | 0.00541 | 0.00283 |
| Solar LCOE 100 MW | 0.01540 | 0.01205 | 0.00938 | 0.00771 |
| Hydrogen production LCOE 100 MW | 0.00972 | 0.00673 | 0.00450 | 0.00374 |
| Ammonia production LCOE | 0.02224 | 0.01832 | 0.01440 | 0.01176 |
| Ammonia transport LCOE | 0.00192 | 0.00172 | 0.00148 | 0.00118 |
| Ammonia storage LCOE | 0.00049 | 0.00034 | 0.00025 | 0.00020 |
| Total LCOE (USD/kWh) | 0.05961 | 0.04715 | 0.03542 | 0.02742 |

Table S5. The cheapest application cost of ammonia to ICE.

| The cheapest application | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| ICE | 0.80 | 0.65 | 0.44 | 0.23 |
| Ammonia storage | 0.01 | 0.07 | 0.05 | 0.04 |
| Ammonia transport (ship) | 0.40 | 0.36 | 0.30 | 0.24 |
| Ammonia production | 4.71 | 3.88 | 3.05 | 2.49 |
| Hydrogen production | 2.59 | 1.79 | 1.19 | 1.00 |
| Renewable electricity | 6.17 | 4.83 | 3.76 | 3.09 |
| Total cost (Million USD) | 14.76 | 11.57 | 8.79 | 7.09 |

Table S6. The cost of ICE-based electricity-ammonia-electricity route in 2023.

| The cost of every part of ammonia supply chain of 2023 (Million USD) | Base value |
| --- | --- |
| Wind energy cost | 10.16 |
| Solar energy cost, 1 MW | 9.12 |
| Solar energy cost, 100 MW | 6.17 |
| Hydrogen production via PEM electrolyzer, 1 MW | 3.22 |
| Hydrogen production via PEM electrolyzer, 100 MW | 3.22 |
| Hydrogen production via alkaline electrolyzer, 1 MW | 2.99 |
| Hydrogen production via alkaline electrolyzer, 100 MW | 2.59 |
| Ammonia production | 4.71 |
| Ammonia transport (ship) | 0.40 |
| Ammonia storage | 0.10 |
| ICE total cost | 0.80 |

# The techno-economic parameters of SOFC-based electricity-ammonia-electricity route.

Table S7. The techno-economic parameters of SOFC-based electricity-ammonia-electricity from 2023 to 2050.

| Techno-economic parameters | Data |  |  |  |
| --- | --- | --- | --- | --- |
| SOFC | 2023 | 2030 | 2040 | 2050 |
| Energy output per year (MWh) | 2710.30 | 2710.30 | 2710.30 | 2710.30 |
| Energy input per year (MWh) | 4927.82 | 4927.82 | 4927.82 | 4927.82 |
| Energy efficiency (%) | 55 | 55 | 55 | 55 |
| Power of SOFC stack (kW) | 700.00 | 700.00 | 700.00 | 700.00 |
| Cost (USD/kW) | 2200.00 | 1617.00 | 1067.00 | 800.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5.5 % | 5.5 % | 5.5 % | 5.5 % |
| Total cost (Million USD) | 4.08 | 3.00 | 1.98 | 1.48 |
| SOFC LCOE (USD/kWh) | 0.05018 | 0.03690 | 0.02435 | 0.01820 |
| Ammonia storage |  |  |  |  |
| Energy output per year (MWh) | 4927.82 | 4927.82 | 4927.82 | 4927.82 |
| Energy input per year (MWh) | 5133.16 | 5133.16 | 5133.16 | 5133.16 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 | 0.00379 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.07 | 0.05 | 0.04 | 0.03 |
| Ammonia storage LCOE (USD/kWh) | 0.00047 | 0.00034 | 0.00027 | 0.0002 |
| Ammonia transport (ship) |  |  |  |  |
| Energy output per year (MWh) | 4927.82 | 4927.82 | 4927.82 | 4927.82 |
| Energy input per year (MWh) | 5133.16 | 5133.16 | 5133.16 | 5133.16 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 | 0.02225 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.29 | 0.26 | 0.22 | 0.18 |
| Ammonia transport LCOE (USD/kWh) | 0.00196 | 0.00176 | 0.00149 | 0.00122 |
| Ammonia production, 0.732 MW |  |  |  |  |
| Total input hydrogen (t) | 177.70 | 177.70 | 177.70 | 177.70 |
| Total output ammonia (t) | 987.17 | 987.17 | 987.17 | 987.17 |
| Total energy output (MWh) | 5133.16 | 5133.16 | 5133.16 | 5133.16 |
| Hydrogen conversion rate (%) | 98 | 98 | 98 | 98 |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 | 0.99 |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 3.42 | 2.82 | 2.21 | 1.81 |
| Ammonia production LCOE (USD/kWh) | 0.02221 | 0.01831 | 0.01435 | 0.01175 |
| Hydrogen production via alkaline electrolyzer, 1.052 MW |  |  |  |  |
| Total energy output (MWh) | 6459.42 | 6459.42 | 6459.42 | 6459.42 |
| Total energy input (MWh) | 9713.44 | 9361.48 | 8909.55 | 8176.48 |
| Energy efficiency (%) | 66 | 68-70 | 70-75 | 78-80 |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 | 385.00 |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 | 275.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 2.17 | 1.65 | 1.30 | 1.01 |
| Total cost (Million USD) 100 MW | 1.88 | 1.30 | 0.87 | 0.72 |
| Hydrogen production LCOE (USD/kWh) 1 MW | 0.01120 | 0.00851 | 0.00671 | 0.00521 |
| Hydrogen production LCOE (USD/kWh) 100 MW | 0.00970 | 0.00671 | 0.00449 | 0.00372 |
| Hydrogen production via PEM electrolyzer, 0.922 MW |  |  |  |  |
| Total energy output (MWh) | 6459.42 | 6459.42 | 6459.42 | 6459.42 |
| Total energy input (MWh) | 8971.42 | 7877.34 | 7689.79 | 7510.95 |
| Energy efficiency (%) | 72 | 82 | 84 | 86 |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 2.34 | 1.65 | 1.14 | 1.01 |
| Total cost (Million USD) 100 MW | 2.34 | 1.65 | 1.14 | 1.01 |
| Hydrogen production LCOE (USD/kWh) | 0.01208 | 0.00851 | 0.00588 | 0.00521 |
| Renewable electricity generation, Solar 5.544 MW, Wind 3.696 MW |  |  |  |  |
| Total energy output (MWh) | 9713.44 | 9713.44 | 9713.44 | 9713.44 |
| Total energy input (MWh) | 48567.19 | 48567.19 | 48567.19 | 48567.19 |
| Energy efficiency (%) | 20 | 20 | 20 | 20 |
| Solar Cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 | 440.00 | 363.00 |
| Solar Cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 | 374.00 | 308.60 |
| Solar Cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 | 308.00 | 253.00 |
| Wind Cost (USD/kW) | 1250.00 | 913.00-1127.50 | 788.00-1070.30 | 638.00-1028.50 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Solar total cost (Million USD) 1 MW | 6.64 | 5.17 | 3.90 | 3.22 |
| Solar total cost (Million USD) 50 MW | 5.64 | 4.40 | 3.32 | 2.74 |
| Solar total cost (Million USD) 100 MW | 4.49 | 3.51 | 2.73 | 2.24 |
| Wind total cost (Million USD) | 7.39 | 5.40-6.67 | 4.66-6.33 | 3.77-6.08 |
| Solar LCOE (USD/kWh) 1 MW | 0.02279 | 0.01774 | 0.01338 | 0.01105 |
| Solar LCOE (USD/kWh) 50 MW | 0.01935 | 0.01510 | 0.01139 | 0.00940 |
| Solar LCOE (USD/kWh) 100 MW | 0.01541 | 0.01205 | 0.00937 | 0.00769 |
| Wind LCOE (USD/kWh) | 0.02536 | 0.01853-0.02289 | 0.01599-0.02172 | 0.01294-0.02086 |
| The lifetime of all equipment is assumed to be 30 years |  |  |  |  |

Table S8. The levelized cost of electricity of different process (1 MW & 100 MW).

| 1 MW | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| SOFC LCOE | 0.05018 | 0.03690 | 0.02435 | 0.01820 |
| Solar LCOE 1 MW | 0.02279 | 0.01774 | 0.01338 | 0.01105 |
| Hydrogen production LCOE 1 MW | 0.01120 | 0.00851 | 0.00671 | 0.00521 |
| Ammonia production LCOE | 0.02221 | 0.01831 | 0.01435 | 0.01175 |
| Ammonia transport LCOE | 0.00196 | 0.00176 | 0.00149 | 0.00122 |
| Ammonia storage LCOE | 0.00047 | 0.00034 | 0.00027 | 0.0002 |
| Total LCOE (USD/kWh) | 0.10881 | 0.08356 | 0.06055 | 0.04763 |
| 100 MW | 2023 | 2030 | 2040 | 2050 |
| SOFC LCOE | 0.05018 | 0.03690 | 0.02435 | 0.01820 |
| Solar LCOE 100 MW | 0.01541 | 0.01205 | 0.00937 | 0.00769 |
| Hydrogen production LCOE 100 MW | 0.00970 | 0.00671 | 0.00449 | 0.00372 |
| Ammonia production LCOE | 0.02221 | 0.01831 | 0.01435 | 0.01175 |
| Ammonia transport LCOE | 0.00196 | 0.00176 | 0.00149 | 0.00122 |
| Ammonia storage LCOE | 0.00047 | 0.00034 | 0.00027 | 0.0002 |
| Total LCOE (USD/kWh) | 0.09993 | 0.07607 | 0.05432 | 0.04278 |

Table S9. The cheapest application cost of ammonia to SOFC.

| The cheapest application | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| SOFC | 4.08 | 3.00 | 1.98 | 1.48 |
| Ammonia storage | 0.07 | 0.05 | 0.04 | 0.03 |
| Ammonia transport (ship) | 0.29 | 0.26 | 0.22 | 0.18 |
| Ammonia production | 3.42 | 2.82 | 2.21 | 1.81 |
| Hydrogen production | 1.88 | 1.30 | 0.87 | 0.72 |
| Renewable electricity | 4.49 | 3.51 | 2.73 | 2.24 |
| Total cost (Million USD) | 14.23 | 10.94 | 8.05 | 6.46 |

Table S10. The cost of every part of ammonia supply chain of 2023 (Million USD).

| The cost of every part of ammonia supply chain of 2023 (Million USD) | Base value |
| --- | --- |
| Wind energy cost | 7.39 |
| Solar energy cost, 1 MW | 6.64 |
| Solar energy cost, 100 MW | 4.49 |
| Hydrogen production via PEM electrolyzer, 1 MW | 2.34 |
| Hydrogen production via PEM electrolyzer, 100 MW | 2.34 |
| Hydrogen production via alkaline electrolyzer, 1 MW | 2.17 |
| Hydrogen production via alkaline electrolyzer, 100 MW | 1.88 |
| Ammonia production | 3.42 |
| Ammonia transport (ship) | 0.29 |
| Ammonia storage | 0.07 |
| SOFC total cost | 4.08 |

# The techno-economic parameters of AEMFC-based electricity-ammonia-electricity route

Table S11. The techno-economic parameters of AEMFC application route from 2023 to 2050.

| Techno-economic parameters | Data |  |  |  |
| --- | --- | --- | --- | --- |
| AEMFC | 2023 | 2030 | 2040 | 2050 |
| Energy output per year (MWh) | 2,710.30 | 2,710.30 | 2,710.30 | 2,710.30 |
| Energy input per year (MWh) | 7743.71 | 7743.71 | 7743.71 | 7743.71 |
| Energy efficiency (%) | 35 | 35 | 35 | 35 |
| Power of AEMFC stack (kW) | 700.00 | 700.00 | 700.00 | 700.00 |
| Cost (USD/kW) | 2211.90 | 1308.80 | 403.60 | 141.80 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5.5 % | 5.5 % | 5.5 % | 5.5 % |
| Total cost (Million USD) | 4.10 | 2.43 | 0.75 | 0.26 |
| AEMFC LCOE (USD/kWh) | 0.05042 | 0.02989 | 0.00922 | 0.00320 |
| Ammonia storage |  |  |  |  |
| Energy output per year (MWh) | 7743.71 | 7743.71 | 7743.71 | 7743.71 |
| Energy input per year (MWh) | 8066.32 | 8066.32 | 8066.32 | 8066.32 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 | 0.00379 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.11 | 0.08 | 0.06 | 0.05 |
| Ammonia storage LCOE (USD/kWh) | 0.00047 | 0.00034 | 0.00026 | 0.00022 |
| Ammonia transport (ship) |  |  |  |  |
| Energy output per year (MWh) | 7743.71 | 7743.71 | 7743.71 | 7743.71 |
| Energy input per year (MWh) | 8066.32 | 8066.32 | 8066.32 | 8066.32 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 | 0.02225 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.45 | 0.41 | 0.34 | 0.28 |
| Ammonia transport LCOE | 0.00194 | 0.00176 | 0.00146 | 0.00121 |
| Ammonia production, 1.151 MW |  |  |  |  |
| Total input hydrogen (t) | 279.24 | 279.24 | 279.24 | 279.24 |
| Total output ammonia (t) | 1551.25 | 1551.25 | 1551.25 | 1551.25 |
| Total energy output (MWh) | 8066.32 | 8066.32 | 8066.32 | 8066.32 |
| Hydrogen conversion rate ( %) | 98 | 98 | 98 | 98 |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 | 0.99 |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 5.38 | 4.43 | 3.48 | 2.85 |
| Ammonia production LCOE (USD/kWh) | 0.02223 | 0.01831 | 0.01438 | 0.01178 |
| Hydrogen production via alkaline electrolyzer, 1.655 MW |  |  |  |  |
| Total energy output (MWh) | 10150.43 | 10150.43 | 10150.43 | 10150.43 |
| Total energy input (MWh) | 15263.83 | 14710.76 | 14000.59 | 12848.64 |
| Energy efficiency (%) | 66 | 68-70 | 70-75 | 78-80 |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 | 385.00 |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 | 275.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.41 | 2.59 | 2.05 | 1.59 |
| Total cost (Million USD) 100 MW | 2.96 | 2.05 | 1.37 | 1.14 |
| Hydrogen production LCOE (USD/kWh) 1 MW | 0.01120 | 0.00851 | 0.00673 | 0.00522 |
| Hydrogen production LCOE (USD/kWh) 100 MW | 0.00972 | 0.00673 | 0.00450 | 0.00374 |
| Hydrogen production via PEM electrolyzer, 1.448 MW |  |  |  |  |
| Total energy output (MWh) | 10150.43 | 10150.43 | 10150.43 | 10150.43 |
| Total energy input (MWh) | 14097.81 | 12378.57 | 12083.84 | 11802.82 |
| Energy efficiency (%) | 72 | 82 | 84 | 86 |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.68 | 2.59 | 1.79 | 1.59 |
| Total cost (Million USD) 100 MW | 3.68 | 2.59 | 1.79 | 1.59 |
| Hydrogen production LCOE (USD/kWh) | 0.01208 | 0.00851 | 0.00588 | 0.00522 |
| Renewable electricity generation, Solar 8.712 MW, Wind 5.808 MW |  |  |  |  |
| Total energy output (MWh) | 15263.83 | 15263.83 | 15263.83 | 15263.83 |
| Total energy input (MWh) | 76319.17 | 76319.17 | 76319.17 | 76319.17 |
| Solar energy efficiency | 15-20 % | 15-20 % | 15-20 % | 15-20 % |
| Wind energy efficiency (%) | 20-40 | 20-40 | 20-40 | 20-40 |
| Solar cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 | 440.00 | 363.00 |
| Solar cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 | 374.00 | 308.60 |
| Solar cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 | 308.00 | 253.00 |
| Wind cost (USD/kW) | 1250.00 | 913.00-1127.50 | 788.00-1070.30 | 638.00-1028.50 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Solar total cost (Million USD) 1 MW | 10.41 | 8.12 | 6.12 | 5.05 |
| Solar total cost (Million USD) 50 MW | 8.85 | 6.90 | 5.21 | 4.30 |
| Solar total cost (Million USD) 100 MW | 7.04 | 5.51 | 4.29 | 3.52 |
| Wind total cost (Million USD) | 11.60 | 8.47-10.46 | 7.31-9.93 | 5.92-9.54 |
| Solar LCOE (USD/kWh) 1 MW | 0.02273 | 0.01773 | 0.01336 | 0.01103 |
| Solar LCOE (USD/kWh) 50 MW | 0.01933 | 0.01507 | 0.01138 | 0.00939 |
| Solar LCOE (USD/kWh) 100 MW | 0.01537 | 0.01203 | 0.00937 | 0.00769 |
| Wind LCOE (USD/kWh) | 0.02533 | 0.01850-0.02285 | 0.01597-0.02169 | 0.01293-0.02084 |
| The lifetime of all equipment are 30 years |  |  |  |  |

Table S12. The levelized cost of electricity of different process (1 MW & 100 MW).

| 1 MW | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| AEMFC LCOE | 0.05042 | 0.02989 | 0.00922 | 0.00320 |
| Solar LCOE 1 MW | 0.02273 | 0.01773 | 0.01336 | 0.01103 |
| Hydrogen production LCOE 1 MW | 0.01120 | 0.00851 | 0.00673 | 0.00522 |
| Ammonia production LCOE | 0.02223 | 0.01831 | 0.01438 | 0.01178 |
| Ammonia transport LCOE | 0.00194 | 0.00176 | 0.00146 | 0.00121 |
| Ammonia storage LCOE | 0.00047 | 0.00034 | 0.00026 | 0.00022 |
| Total LCOE (USD/kWh) | 0.10899 | 0.07654 | 0.04541 | 0.03266 |
| 100 MW | 2023 | 2030 | 2040 | 2050 |
| AEMFC LCOE | 0.05042 | 0.02989 | 0.00922 | 0.00320 |
| Solar LCOE 100 MW | 0.01537 | 0.01203 | 0.00937 | 0.00769 |
| Hydrogen production LCOE 100 MW | 0.00972 | 0.00673 | 0.00450 | 0.00374 |
| Ammonia production LCOE | 0.02223 | 0.01831 | 0.01438 | 0.01178 |
| Ammonia transport LCOE | 0.00194 | 0.00176 | 0.00146 | 0.00121 |
| Ammonia storage LCOE | 0.00047 | 0.00034 | 0.00026 | 0.00022 |
| Total LCOE (USD/kWh) | 0.10015 | 0.06906 | 0.03919 | 0.02784 |

Table S13. The cheapest application cost of ammonia to AEMFC.

| The cheapest application | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| AEMFC | 4.10 | 2.43 | 0.75 | 0.26 |
| Ammonia storage | 0.11 | 0.08 | 0.06 | 0.05 |
| Ammonia transport | 0.45 | 0.41 | 0.34 | 0.28 |
| Ammonia production | 5.38 | 4.43 | 3.48 | 2.85 |
| Hydrogen production | 2.96 | 2.05 | 1.37 | 1.14 |
| Renewable electricity | 7.04 | 5.51 | 4.29 | 3.52 |
| Total cost (Million USD) | 20.05 | 14.90 | 10.29 | 8.10 |

Table S14. The cost of every part of ammonia supply chain of 2023 (Million USD).

| The cost of every part of ammonia supply chain of 2023 (Million USD) | Base value |
| --- | --- |
| Wind energy cost | 11.60 |
| Solar energy cost, 1 MW | 10.41 |
| Solar energy cost, 100 MW | 7.04 |
| Hydrogen production via PEM electrolyzer, 1 MW | 3.68 |
| Hydrogen production via PEM electrolyzer, 100 MW | 3.68 |
| Hydrogen production via alkaline electrolyzer, 1 MW | 3.41 |
| Hydrogen production via alkaline electrolyzer, 100 MW | 2.96 |
| Ammonia production | 5.38 |
| Ammonia transport (ship) | 0.45 |
| Ammonia storage | 0.11 |
| AEMFC total cost | 4.10 |

# 5. The techno-economic parameters of PEMFC-based electricity-ammonia-electricity route

Table S15. The techno-economic parameters of PEMFC (Cracker) application route from 2023 to 2050.

| Techno-economic parameters | Data |  |  |  |
| --- | --- | --- | --- | --- |
| PEMFC Stack | 2023 | 2030 | 2040 | 2050 |
| Energy output per year (MWh) | 2710.30 | 2710.30 | 2710.30 | 2710.30 |
| Energy input per year (MWh) | 5420.60 | 5420.60 | 5420.60 | 5420.60 |
| Energy efficiency (%) | 50 | 50 | 50 | 50 |
| Total hydrogen input (t) | 162.78 | 162.78 | 162.78 | 162.78 |
| Power of PEMFC stack (kW) | 700.00 | 700.00 | 700.00 | 700.00 |
| Cost (USD/kW) | 166.00 | 80.00 | 70.00 | 60.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 0.29 | 0.14 | 0.12 | 0.11 |
| PEMFC LCOE (USD/kWh) | 0.00357 | 0.00172 | 0.00148 | 0.00135 |
| Ammonia cracking system |  |  |  |  |
| Energy output per year (MWh) | 5420.60 | 5420.60 | 5420.60 | 5420.60 |
| Energy input per year (MWh) | 7275.97 | 7275.97 | 7275.97 | 7275.97 |
| Total ammonia input (t) | 1229.70 | 1229.70 | 1229.70 | 1229.70 |
| Energy efficiency ( %) | 70-79 | 70-79 | 70-79 | 70-79 |
| Cost (USD/MWh) | 153.16 | 153.16 | 153.16 | 153.16 |
| O & M cost (USD/kW) (of the CAPEX cost) | 1 % | 1 % | 1 % | 1 % |
| Total cost (Million USD) | 1.08 | 1.08 | 1.08 | 1.08 |
| Cracker LCOE (USD/kWh) | 0.00664 | 0.00664 | 0.00664 | 0.00664 |
| PEMFC with cracker total cost (Million USD) | 1.37 | 1.22 | 1.20 | 1.19 |
| PEMFC with cracker LCOE (USD/kWh) | 0.01021 | 0.00836 | 0.00812 | 0.00799 |
| Ammonia storage |  |  |  |  |
| Energy output per year (MWh) | 7132.40 | 7132.40 | 7132.40 | 7132.40 |
| Energy input per year (MWh) | 7429.60 | 7429.60 | 7429.60 | 7429.60 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 | 0.00379 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.10 | 0.07 | 0.06 | 0.04 |
| Ammonia storage LCOE (USD/kWh) | 0.00047 | 0.00033 | 0.00028 | 0.00019 |
| Ammonia transport (ship) |  |  |  |  |
| Energy output per year (MWh) | 7132.40 | 7132.40 | 7132.40 | 7132.40 |
| Energy input per year (MWh) | 7429.60 | 7429.60 | 7429.60 | 7429.60 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 | 0.02225 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.41 | 0.37 | 0.32 | 0.25 |
| Ammonia transport LCOE (USD/kWh) | 0.00192 | 0.00173 | 0.00150 | 0.00117 |
| Ammonia production, 1.060 MW |  |  |  |  |
| Total input hydrogen (t) | 257.20 | 257.20 | 257.20 | 257.20 |
| Total output ammonia (t) | 1428.80 | 1428.80 | 1428.80 | 1428.80 |
| Total energy output (MWh) | 7429.60 | 7429.60 | 7429.60 | 7429.60 |
| Hydrogen conversion rate (%) | 98 | 98 | 98 | 98 |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 | 0.99 |
| O & M cost (Million USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 4.96 | 4.08 | 3.21 | 2.62 |
| Ammonia production LCOE (USD/kWh) | 0.02225 | 0.01831 | 0.01440 | 0.01175 |
| Hydrogen production via alkaline electrolyzer, 1.525 MW |  |  |  |  |
| Total energy output (MWh) | 9349.20 | 9349.20 | 9349.20 | 9349.20 |
| Total energy input (MWh) | 14058.95 | 13549.57 | 12895.45 | 11834.43 |
| Energy efficiency (%) | 66 | 68-70 | 70-75 | 78-80 |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 | 385.00 |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 | 275.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.15 | 2.39 | 1.89 | 1.47 |
| Total cost (Million USD) 100 MW | 2.73 | 1.89 | 1.26 | 1.05 |
| Hydrogen production LCOE (USD/kWh) 1 MW | 0.01123 | 0.00852 | 0.00674 | 0.00524 |
| Hydrogen production LCOE (USD/kWh) 100 MW | 0.00973 | 0.00674 | 0.00449 | 0.00374 |
| Hydrogen production via PEM electrolyzer, 1.334 MW |  |  |  |  |
| Total energy output (MWh) | 9349.20 | 9349.20 | 9349.20 | 9349.20 |
| Total energy input (MWh) | 12985.00 | 11401.46 | 11130.00 | 10871.16 |
| Energy efficiency (%) | 72 | 82 | 84 | 86 |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.39 | 2.38 | 1.65 | 1.47 |
| Total cost (Million USD) 100 MW | 3.39 | 2.38 | 1.65 | 1.47 |
| Hydrogen production LCOE (USD/kWh) | 0.01209 | 0.00849 | 0.00588 | 0.00524 |
| Renewable electricity generation, Solar 8.025 MW, Wind 5.350 MW |  |  |  |  |
| Total energy output (MWh) | 14058.98 | 14058.98 | 14058.98 | 14058.98 |
| Total energy input (MWh) | 70294.90 | 70294.90 | 70294.90 | 70294.90 |
| Energy efficiency (%) | 20 | 20 | 20 | 20 |
| Solar cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 | 440.00 | 363.00 |
| Solar cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 | 374.00 | 308.60 |
| Solar cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 | 308.60 | 253.00 |
| Wind cost (USD/kW) | 1250.00 | 913.00-1127.50 | 788.00-1070.30 | 638.00-1028.50 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Solar total cost (Million USD) 1 MW | 9.60 | 7.49 | 5.65 | 4.66 |
| Solar total cost (Million USD) 50 MW | 8.16 | 6.36 | 4.80 | 3.96 |
| Solar total cost (Million USD) 100 MW | 6.50 | 5.08 | 3.96 | 3.25 |
| Wind total cost (Million USD) | 10.70 | 7.82-9.65 | 6.75-9.16 | 5.46-8.80 |
| Solar LCOE 1 MW (USD/kWh) | 0.02276 | 0.01776 | 0.0134 | 0.01105 |
| Solar LCOE 50 MW (USD/kWh) | 0.01935 | 0.01508 | 0.01138 | 0.00939 |
| Solar LCOE 100 MW (USD/kWh) | 0.01541 | 0.01204 | 0.00939 | 0.00771 |
| Wind LCOE (USD/kWh) | 0.02537 | 0.01854-0.02288 | 0.01600-0.02172 | 0.01295-0.02086 |
| The lifetime of all equipment are 30 years |  |  |  |  |

Table S16. The levelized cost of electricity of different process (1 MW & 100 MW).

| 1 MW | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| PEMFC with cracker LCOE | 0.01021 | 0.00836 | 0.00812 | 0.00799 |
| Solar LCOE 1 MW | 0.02276 | 0.01776 | 0.01340 | 0.01105 |
| Hydrogen production LCOE 1 MW | 0.01123 | 0.00852 | 0.00674 | 0.00524 |
| Ammonia production LCOE | 0.02225 | 0.01831 | 0.01440 | 0.01175 |
| Ammonia transport LCOE | 0.00192 | 0.00173 | 0.00150 | 0.00117 |
| Ammonia storage LCOE | 0.00047 | 0.00033 | 0.00028 | 0.00019 |
| Total LCOE (USD/kWh) | 0.06884 | 0.05501 | 0.04444 | 0.03739 |
| 100 MW | 2023 | 2030 | 2040 | 2050 |
| PEMFC with cracker LCOE | 0.01021 | 0.00836 | 0.00812 | 0.00799 |
| Solar LCOE 100 MW | 0.01541 | 0.01204 | 0.00939 | 0.00771 |
| Hydrogen production LCOE 100 MW | 0.00973 | 0.00674 | 0.00449 | 0.00374 |
| Ammonia production LCOE | 0.02225 | 0.01831 | 0.01440 | 0.01175 |
| Ammonia transport LCOE | 0.00192 | 0.00173 | 0.00150 | 0.00117 |
| Ammonia storage LCOE | 0.00047 | 0.00033 | 0.00028 | 0.00019 |
| Total LCOE (USD/kWh) | 0.05999 | 0.04751 | 0.03818 | 0.03255 |

Table S17. The cheapest application cost of ammonia to PEMFC with cracker.

| The cheapest application | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| PEMFC with Cracker | 1.37 | 1.22 | 1.20 | 1.19 |
| Ammonia storage | 0.10 | 0.07 | 0.06 | 0.04 |
| Ammonia transport (ship) | 0.41 | 0.37 | 0.32 | 0.25 |
| Ammonia production | 4.96 | 4.08 | 3.21 | 2.62 |
| Hydrogen production | 2.73 | 1.89 | 1.26 | 1.05 |
| Renewable electricity | 6.50 | 5.08 | 3.96 | 3.25 |
| Total cost (Million USD) | 16.08 | 12.71 | 10.00 | 8.41 |

Table S18. The cost of every part of ammonia supply chain of 2023 (Million USD).

| The cost of every part of ammonia supply chain of 2023 (Million USD) | Base value |
| --- | --- |
| Wind energy cost | 10.70 |
| Solar energy cost, 1 MW | 9.60 |
| Solar energy cost, 100 MW | 6.50 |
| Hydrogen production via PEM electrolyzer, 1 MW | 3.39 |
| Hydrogen production via PEM electrolyzer, 100 MW | 3.39 |
| Hydrogen production via alkaline electrolyzer, 1 MW | 3.15 |
| Hydrogen production via alkaline electrolyzer, 100 MW | 2.73 |
| Ammonia production | 4.96 |
| Ammonia transport (ship) | 0.42 |
| Ammonia storage | 0.11 |
| Ammonia cracker cost | 1.08 |
| PEMFC with cracker total cost | 1.37 |

# 6. The techno-economic parameters of SOFC-based electricity-ammonia-electricity route

Table S19. The techno-economic parameters of SOFC (Cracker) application route from 2023 to 2050.

| Techno-economic parameters | Data |  |  |  |
| --- | --- | --- | --- | --- |
| SOFC stack | 2023 | 2030 | 2040 | 2050 |
| Energy output per year (MWh) | 2710.30 | 2710.30 | 2710.30 | 2710.30 |
| Energy input per year (MWh) | 4927.82 | 4927.82 | 4927.82 | 4927.82 |
| Energy efficiency (%) | 55 | 55 | 55 | 55 |
| Power of SOFC stack (kW) | 700.00 | 700.00 | 700.00 | 700.00 |
| Cost (USD/kW) | 2200.00 | 1617.00 | 1067.00 | 800.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5.5 % | 5.5 % | 5.5 % | 5.5 % |
| Total cost (Million USD) | 4.08 | 3.00 | 1.98 | 1.48 |
| SOFC LCOE (USD/kWh) | 0.05018 | 0.03690 | 0.02435 | 0.01820 |
| Ammonia cracking system |  |  |  |  |
| Energy output per year (MWh) | 4927.82 | 4927.82 | 4927.82 | 4927.82 |
| Energy input per year (MWh) | 6483.97 | 6614.52 | 6614.52 | 6614.52 |
| Energy efficiency (%) | 70-79 | 70-79 | 70-79 | 70-79 |
| Cost (USD/MWh) | 153.16 | 153.16 | 153.16 | 153.16 |
| O & M cost (USD/kW) (of the CAPEX cost) | 1 % | 1 % | 1 % | 1 % |
| Total cost (Million USD) | 0.98 | 0.98 | 0.98 | 0.98 |
| Cracker LCOE (USD/kWh) | 0.00664 | 0.00664 | 0.00664 | 0.00664 |
| SOFC with cracker total cost (Million USD) | 5.06 | 3.98 | 2.96 | 2.46 |
| SOFC with cracker LCOE (USD/kWh) | 0.05682 | 0.04354 | 0.03099 | 0.02484 |
| Ammonia storage |  |  |  |  |
| Energy output per year (MWh) | 6483.97 | 6483.97 | 6483.97 | 6483.97 |
| Energy input per year (MWh) | 6754.14 | 6754.14 | 6754.14 | 6754.14 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| CAPEX cost (USD/kWh) | 0.00910 | 0.00610 | 0.00485 | 0.00379 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.09 | 0.06 | 0.05 | 0.04 |
| Ammonia storage LCOE (USD/kWh) | 0.00046 | 0.00031 | 0.00026 | 0.00021 |
| Ammonia transport (ship) |  |  |  |  |
| Energy output per year (MWh) | 6483.97 | 6483.97 | 6483.97 | 6483.97 |
| Energy input per year (MWh) | 6754.14 | 6754.14 | 6754.14 | 6754.14 |
| Energy loss (%) | 4 | 4 | 4 | 4 |
| Cost (USD/kWh) | 0.03636 | 0.03272 | 0.02782 | 0.02225 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Total cost (Million USD) | 0.38 | 0.34 | 0.29 | 0.23 |
| Ammonia transport LCOE | 0.00195 | 0.00175 | 0.00149 | 0.00118 |
| Ammonia production, 0.964 MW |  |  |  |  |
| Total input hydrogen (t) | 233.82 | 233.82 | 233.82 | 233.82 |
| Total output ammonia (t) | 1298.90 | 1298.90 | 1298.90 | 1298.90 |
| Total energy output (MWh) | 6754.14 | 6754.14 | 6754.14 | 6754.14 |
| Hydrogen conversion rate (%) | 98 | 98 | 98 | 98 |
| Cost (Million USD/MW NH3) | 1.87 | 1.54 | 1.21 | 0.99 |
| O & M cost (Mllion USD/MW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) | 4.51 | 3.71 | 2.92 | 2.39 |
| Ammonia production LCOE (USD/kWh) | 0.02226 | 0.01831 | 0.01441 | 0.01180 |
| Hydrogen production via alkaline electrolyzer, 1.386 MW |  |  |  |  |
| Total energy output (MWh) | 8499.21 | 8499.21 | 8499.21 | 8499.21 |
| Total energy input (MWh) | 12780.80 | 12317.70 | 11723.05 | 10758.50 |
| Energy efficiency ( %) | 66 | 68-70 | 70-75 | 78-80 |
| Cost (USD/kW) 1 MW | 825.00 | 627.00 | 495.00 | 385.00 |
| Cost (USD/kW) 100 MW | 715.00 | 495.00 | 330.00 | 275.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 2.86 | 2.17 | 1.72 | 1.33 |
| Total cost (Million USD) 100 MW | 2.48 | 1.72 | 1.14 | 0.95 |
| Hydrogen production LCOE (USD/kWh) 1 MW | 0.01122 | 0.00851 | 0.00675 | 0.00522 |
| Hydrogen production LCOE (USD/kWh) 100 MW | 0.00973 | 0.00675 | 0.00447 | 0.00373 |
| Hydrogen production via PEM electrolyzer, 1.213 MW |  |  |  |  |
| Total energy output (MWh) | 8499.21 | 8499.21 | 8499.21 | 8499.21 |
| Total energy input (MWh) | 11804.46 | 10364.90 | 10118.11 | 9882.81 |
| Energy efficiency (%) | 72 | 82 | 84 | 86 |
| Cost (USD/kW) 1 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| Cost (USD/kW) 100 MW | 1017.50 | 715.00 | 495.00 | 440.00 |
| O & M cost (USD/kW) (of the CAPEX cost) | 5 % | 5 % | 5 % | 5 % |
| Total cost (Million USD) 1 MW | 3.09 | 2.17 | 1.50 | 1.33 |
| Total cost (Million USD) 100 MW | 3.09 | 2.17 | 1.50 | 1.33 |
| Hydrogen production LCOE (USD/kWh) | 0.01212 | 0.00851 | 0.00588 | 0.00522 |
| Renewable electricity generation, Solar 7.295 MW, Wind 4.863 MW |  |  |  |  |
| Total energy output (MWh) | 12780.80 | 12780.80 | 12780.80 | 12780.80 |
| Total energy input (MWh) | 63904.02 | 63904.02 | 63904.02 | 63904.02 |
| Energy efficiency (%) | 20 | 20 | 20 | 20 |
| Solar cost (USD/kW) Industrial, 1 MW | 748.00 | 583.00 | 440.00 | 363.00 |
| Solar cost (USD/kW) Utility-scale, 50 MW | 635.80 | 495.60 | 374.00 | 308.60 |
| Solar cost (USD/kW) Utility-scale, 100 MW | 506.00 | 396.00 | 308.60 | 253.00 |
| Wind cost (USD/kW) | 1250.00 | 913.00-1127.50 | 788.00-1070.30 | 638.00-1028.50 |
| O & M cost (USD/kW) (of the CAPEX cost) | 2 % | 2 % | 2 % | 2 % |
| Solar total cost (Million USD) 1 MW | 8.73 | 6.80 | 5.14 | 4.24 |
| Solar total cost (Million USD) 50 MW | 7.42 | 5.78 | 4.37 | 3.60 |
| Solar total cost (Million USD) 100 MW | 5.91 | 4.62 | 3.60 | 2.95 |
| Wind total cost (Million USD) | 9.73 | 7.10-8.77 | 6.13-8.33 | 4.96-8.00 |
| Solar LCOE (USD/kWh) 1 MW | 0.02277 | 0.01773 | 0.01341 | 0.01106 |
| Solar LCOE (USD/kWh) 50 MW | 0.01935 | 0.01507 | 0.0114 | 0.00939 |
| Solar LCOE (USD/kWh) 100 MW | 0.01541 | 0.01205 | 0.00939 | 0.00769 |
| Wind LCOE (USD/kWh) | 0.02538 | 0.01852-0.02287 | 0.01599-0.02173 | 0.01294-0.02086 |
| The lifetime of all equipment are 30 years |  |  |  |  |

Table S20. The levelized cost of electricity of different process (1 MW & 100 MW).

| 1 MW | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| SOFC with cracker LCOE | 0.05682 | 0.04354 | 0.03099 | 0.02484 |
| Solar LCOE 1 MW | 0.02277 | 0.01773 | 0.01341 | 0.01106 |
| Hydrogen production LCOE 1 MW | 0.01122 | 0.00851 | 0.00675 | 0.00522 |
| Ammonia production LCOE | 0.02226 | 0.01831 | 0.01441 | 0.01180 |
| Ammonia transport LCOE | 0.00195 | 0.00175 | 0.00149 | 0.00118 |
| Ammonia storage LCOE | 0.00046 | 0.00031 | 0.00026 | 0.00021 |
| Total LCOE (USD/kWh) | 0.11548 | 0.09015 | 0.06731 | 0.05431 |
| 100 MW | 2023 | 2030 | 2040 | 2050 |
| SOFC with cracker LCOE | 0.05682 | 0.04354 | 0.03099 | 0.02484 |
| Solar LCOE 100 MW | 0.01541 | 0.01205 | 0.00939 | 0.00769 |
| Hydrogen production LCOE 100 MW | 0.00973 | 0.00675 | 0.00447 | 0.00373 |
| Ammonia production LCOE | 0.02226 | 0.01831 | 0.01441 | 0.01180 |
| Ammonia transport LCOE | 0.00195 | 0.00175 | 0.00149 | 0.00118 |
| Ammonia storage LCOE | 0.00046 | 0.00031 | 0.00026 | 0.00021 |
| Total LCOE (USD/kWh) | 0.10663 | 0.08271 | 0.06101 | 0.04945 |

Table S21. The cheapest application cost of ammonia to SOFC with cracker.

| The cheapest application | 2023 | 2030 | 2040 | 2050 |
| --- | --- | --- | --- | --- |
| SOFC with Cracker | 5.06 | 3.98 | 2.96 | 2.46 |
| Ammonia storage | 0.09 | 0.06 | 0.05 | 0.04 |
| Ammonia transport (ship) | 0.38 | 0.34 | 0.29 | 0.23 |
| Ammonia production | 4.51 | 3.71 | 2.92 | 2.39 |
| Hydrogen production | 2.48 | 1.72 | 1.14 | 0.95 |
| Renewable electricity | 5.91 | 4.62 | 3.60 | 2.95 |
| Total cost (Million USD) | 18.43 | 14.43 | 10.96 | 9.02 |

Table S22. The cost of every part of ammonia supply chain of 2023 (Million USD).

| The cost of every part of ammonia supply chain of 2023 (Million USD) | Base value |
| --- | --- |
| Wind energy cost | 9.73 |
| Solar energy cost, 1 MW | 8.73 |
| Solar energy cost, 100 MW | 5.91 |
| Hydrogen production via PEM electrolyzer, 1 MW | 3.09 |
| Hydrogen production via PEM electrolyzer, 100 MW | 3.09 |
| Hydrogen production via alkaline electrolyzer, 1 MW | 2.86 |
| Hydrogen production via alkaline electrolyzer, 100 MW | 2.48 |
| Ammonia production | 4.51 |
| Ammonia transport (ship) | 0.38 |
| Ammonia storage | 0.09 |
| Ammonia cracker cost | 0.98 |
| SOFC with cracker total cost | 5.06 |

# 7. The energy conversion efficiency of the five application routes.

Table S23. The efficiency of different application route from 2023 to 2050.

| Five cases efficiency (%) | 2023 |  | 2030 |  | 2040 |  | 2050 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonia-ICE | Basic | Range | Basic | Range | Basic | Range | Basic | Range |
| Hydrogen production via alkaline electrolysis | 66.50 % |  | 68.00 % | 2.00 % | 70.00 % | 5.00 % | 78.00 % | 2.00 % |
| Ammonia production | 65.17 % |  | 66.64 % |  | 68.60 % |  | 76.44 % |  |
| Ammonia transport (ship) | 62.56 % |  | 63.97 % |  | 65.86 % |  | 73.38 % |  |
| Ammonia storage | 60.06 % |  | 61.42 % |  | 63.22 % |  | 70.45 % |  |
| Ammonia cracking system total cost | 42.04 % | 5.41 % | 42.99 % | 5.53 % | 44.26 % | 5.69 % | 49.31 % | 6.34 % |
| ICE | 16.82 % | 3.78 % | 17.20 % | 3.87 % | 17.70 % | 3.98 % | 19.73 % | 4.44 % |
| Ammonia-SOFC | Basic | Range | Basic | Range | Basic | Range | Basic | Range |
| Hydrogen production via alkaline electrolysis | 66.50 % |  | 68.00 % | 2.00 % | 70.00 % | 5.00 % | 78.00 % | 2.00 % |
| Ammonia production | 65.17 % |  | 66.64 % |  | 68.60 % |  | 76.44 % |  |
| Ammonia transport (ship) | 62.56 % |  | 63.97 % |  | 65.86 % |  | 73.38 % |  |
| Ammonia storage | 60.06 % |  | 61.42 % |  | 63.22 % |  | 70.45 % |  |
| SOFC | 33.03 % |  | 33.78 % |  | 34.77 % |  | 38.75 % |  |
| Ammonia-AEMFC | Basic | Range | Basic | Range | Basic | Range | Basic | Range |
| Hydrogen production via alkaline electrolysis | 66.50 % |  | 68.00 % | 2.00 % | 70.00 % | 5.00 % | 78.00 % | 2.00 % |
| Ammonia production | 65.17 % |  | 66.64 % |  | 68.60 % |  | 76.44 % |  |
| Ammonia transport (ship) | 62.56 % |  | 63.97 % |  | 65.86 % |  | 73.38 % |  |
| Ammonia storage | 60.06 % |  | 61.42 % |  | 63.22 % |  | 70.45 % |  |
| AEMFC | 10.21 % |  | 21.50 % |  | 26.55 % |  | 31.70 % |  |
| Ammonia-H2-PEMFC | Basic | Range | Basic | Range | Basic | Range | Basic | Range |
| Hydrogen production via alkaline electrolysis | 66.50 % |  | 68.00 % | 2.00 % | 70.00 % | 5.00 % | 78.00 % | 2.00 % |
| Ammonia production | 65.17 % |  | 66.64 % |  | 68.60 % |  | 76.44 % |  |
| Ammonia transport (ship) | 62.56 % |  | 63.97 % |  | 65.86 % |  | 73.38 % |  |
| Ammonia storage | 60.06 % |  | 61.42 % |  | 63.22 % |  | 70.45 % |  |
| Ammonia cracking system total cost | 42.04 % | 5.41 % | 42.99 % | 5.53 % | 44.26 % | 5.69 % | 49.31 % | 6.34 % |
| PEMFC with cracker | 21.02 % | 3.78 % | 21.50 % | 3.87 % | 22.13 % | 3.98 % | 24.66 % | 4.44 % |
| Ammonia-H2-SOFC | Basic | Range | Basic | Range | Basic | Range | Basic | Range |
| Hydrogen production via alkaline electrolysis | 66.50 % |  | 68.00 % | 2.00 % | 70.00 % | 5.00 % | 78.00 % | 2.00 % |
| Ammonia production | 65.17 % |  | 66.64 % |  | 68.60 % |  | 76.44 % |  |
| Ammonia transport (ship) | 62.56 % |  | 63.97 % |  | 65.86 % |  | 73.38 % |  |
| Ammonia storage | 60.06 % |  | 61.42 % |  | 63.22 % |  | 70.45 % |  |
| Ammonia cracking system total cost | 42.04 % | 5.41 % | 42.99 % | 5.53 % | 44.26 % | 5.69 % | 49.31 % | 6.34 % |
| SOFC with cracker | 23.12 % | 3.78 % | 23.64 % | 3.87 % | 24.34 % | 3.98 % | 27.12 % | 4.44 % |

Table S24. The total cost of different application route from 2023 to 2050.

| Five cases costs (Million USD) | 2023 | 2030 | 2040 | 2050 | decrease rate |
| --- | --- | --- | --- | --- | --- |
| Ammonia to ICE | 14.76 | 11.57 | 8.79 | 7.09 | 51.96 % |
| Ammonia to SOFC | 14.23 | 10.94 | 8.05 | 6.46 | 54.60 % |
| Ammonia to AEMFC | 20.05 | 14.90 | 10.29 | 8.10 | 59.60 % |
| Ammonia to PEMFC (with cracker) | 16.08 | 12.71 | 10.00 | 8.41 | 47.70 % |
| Ammonia to SOFC (with cracker) | 18.43 | 14.43 | 10.96 | 9.02 | - % |

# 8. The section cost of the electricity to ammonia of the fuel cells-based application.

SOFC-based electricity to ammonia is the SOFC as the terminal application to convert fuels to clean electricity, refers to the parts of ammonia value chain containing renewable electricity generation, green hydrogen production, and green ammonia production.

Figure S1. Cost analysis of SOFC-based electricity to ammonia

AEMFC-based electricity to ammonia is the AEMFC as the terminal application to convert fuels to clean electricity, refers to the parts of ammonia value chain containing renewable electricity generation, green hydrogen production, and green ammonia production.

Figure S2. Cost analysis of AEMFC-based electricity to ammonia

SOFC (cracker)-based electricity to ammonia is the SOFC with cracker system as the terminal application to convert fuels to clean electricity, refers to the parts of ammonia value chain containing renewable electricity generation, green hydrogen production, and green ammonia production.

Figure S3. Cost analysis of SOFC (cracker) -based electricity to ammonia

# 9. The size effect on the system cost of ICE supply chain

ICE-based electricity to ammonia to hydrogen to electricity route is the ICE and generator system as the terminal application to convert fuels to clean electricity, refers to the parts of ammonia value chain containing renewable electricity generation, green hydrogen production, green ammonia production, ammonia transportation, ammonia storage, and clean electricity generation.

Figure S4. Size effect on the system cost of ICE-based electricity-ammonia-hydrogen-electricity route (1 MW & 100 MW)

# 10. Comparative Analysis: Ammonia ICE vs. PEMFC-Based Electricity Generation

(1) Key Specifications

| Parameter | Ammonia ICE [35, 36] | PEMFC (with NH₃ Cracking) [37-39] |
| --- | --- | --- |
| Efficiency (LHV) | 35–45% | 45–55% (system) |
| Part-Load Efficiency | Drops by 10–20% | Maintains >90% of peak efficiency |
| Startup Time | Seconds–minutes | Minutes (cracker preheat needed) |
| Capital Cost ($/kW) | 800–1,500 | 2,000–4,000 (incl. cracker) |
| Lifetime (hours) | 20,000–30,000 | 40,000–60,000 (PEMFC) |

(2) System-Level Comparison

| Parameter | Ammonia ICE [35, 36] | PEMFC (with NH₃ Cracking) [37-39] |
| --- | --- | --- |
| System Efficiency | Lower (35–45%) | Higher (45–55%) |
| Scalability | High (MW scale) | Modular but cost-limited |
| Emissions | NOx-intensive | Near-zero (with clean H₂) |
| Cost ($/kWh) | Lower capex, higher Opex (SCR/EGR) | Higher capex, lower Opex |
| Technology Maturity | Commercial (marine/stationary) | Emerging (needs cracker integration) |

# References

[1]	Blanco D, Rivera Y, Berna-Escriche C, Muñoz-Cobo JL. Economy decarbonization using green hydrogen and electricity, forecasts and sensitivity analysis for the Canarian Islands in 2040. J Energy Storage. 2024;80:110232.

[2]	Elkadeem MR, Kotb KM, Sharshir SW, Hamada MA, Kabeel AE, Gabr IK, et al. Optimize and analyze a large-scale grid-tied solar PV-powered SWRO system for sustainable water-energy nexus. Desalination. 2024;579:117440.

[3]	Fridleifsson IB. Geothermal energy for the benefit of the people. Renew Sustain Energy Rev. 2001;5:299-312.

[4]	Hil Baky MA, Rahman MM, Islam AKMS. Development of renewable energy sector in Bangladesh: Current status and future potentials. Renew Sustain Energy Rev. 2017;73:1184-97.

[5]	Logan BE, Rossi R, Baek G, Shi L, O’Connor J, Peng W. Energy Use for Electricity Generation Requires an Assessment More Directly Relevant to Climate Change. ACS Energy Lett. 2020;5:3514-7.

[6]	Yuan X, Su C-W, Umar M, Shao X, LobonŢ O-R. The race to zero emissions: Can renewable energy be the path to carbon neutrality? J Environ Manage. 2022;308:114648.

[7]	Zhai W, Ma Y, Chen D, Ho JC, Dai Z, Qu Y. Recent progress on the long-term stability of hydrogen evolution reaction electrocatalysts. InfoMat. 2022;4:e12357.

[8]	Armijo J, Philibert C. Flexible production of green hydrogen and ammonia from variable solar and wind energy: Case study of Chile and Argentina. Int J Hydrogen Energy. 2020;45:1541-58.

[9]	Breyer C. Low-cost solar power enables a sustainable energy industry system. Proc Natl Acad Sci. 2021;118:e2116940118.

[10]	Vartiainen E, Masson G, Breyer C. The true competitiveness of solar PV: a European case study. Proceedings of the Annual Conference of the European Technology and Innovation Platform Photovoltaics-PV Manufacturing in Europe,(ETIP PV), Brussels, Belgium2017. p. 18-9.

[11]	Ikäheimo J, Kiviluoma J, Weiss R, Holttinen H. Power-to-ammonia in future North European 100 % renewable power and heat system. Int J Hydrogen Energy. 2018;43:17295-308.

[12]	Bertuccioli L, Chan A, Hart D, Lehner F, Madden B, Standen E. Development of water electrolysis in the European Union. 2014.

[13]	Council H. Hydrogen Insights 2023: An Update on the State of the Global Hydrogen Economy, with a Deep Dive into North America. 2023. 2023.

[14]	Energinet TDEAa. Technology Data – Renewable fuels2017.

[15]	Chi J, Yu H. Water electrolysis based on renewable energy for hydrogen production. Chin J Catal. 2018;39:390-4.

[16]	Hu K, Fang J, Ai X, Huang D, Zhong Z, Yang X, et al. Comparative study of alkaline water electrolysis, proton exchange membrane water electrolysis and solid oxide electrolysis through multiphysics modeling. Appl Energy. 2022;312:118788.

[17]	Wang Y, Wen C, Tu J, Zhan Z, Zhang B, Liu Q, et al. The multi-scenario projection of cost reduction in hydrogen production by proton exchange membrane (PEM) water electrolysis in the near future (2020–2060) of China. Fuel. 2023;354:129409.

[18]	Xue F, Su J, Li P, Zhang Y. Application of Proton Exchange Membrane Electrolysis of Water Hydrogen Production Technology in Power Plant. IOP Conf Ser Earth Environ Sci. 2021;631:012079.

[19]	Giddey S, Badwal SPS, Kulkarni A. ChemInform Abstract: Review of Electrochemical Ammonia Production Technologies and Materials. Cheminf. 2014;45.

[20]	Wang B, Li T, Gong F, Othman MHD, Xiao R. Ammonia as a green energy carrier: Electrochemical synthesis and direct ammonia fuel cell - a comprehensive review. Fuel Process Technol. 2022;235:107380.

[21]	Martín JG. The future of hydrogen: Seizing todays opportunities. Eco Industria. 2022:183-4.

[22]	Boretti A, Castelletto S. NH3 Prospects in Combustion Engines and Fuel Cells for Commercial Aviation by 2030. ACS Energy Lett. 2022;7:2557-64.

[23]	Chatterjee S, Parsapur RK, Huang K-W. Limitations of Ammonia as a Hydrogen Energy Carrier for the Transportation Sector. ACS Energy Lett. 2021;6:4390-4.

[24]	Kistner L, Bensmann A, Minke C, Hanke-Rauschenbach R. Comprehensive techno-economic assessment of power technologies and synthetic fuels under discussion for ship applications. Renew Sustain Energy Rev. 2023;183:113459.

[25]	Körner A, Tam C, Bennett S, Gagné J. Technology roadmap-hydrogen and fuel cells. International Energy Agency (IEA): Paris, France. 2015:1-29.

[26]	Rameez PV, Mohamed Ibrahim M. A comprehensive review on the utilization of hydrogen in low temperature combustion strategies: Combustion, performance and emission attributes. J Energy Inst. 2024;113:101511.

[27]	Wang J, Wang H, Fan Y. Techno-Economic Challenges of Fuel Cell Commercialization. Eng. 2018;4:352-60.

[28]	Sgroi MF, Zedde F, Barbera O, Stassi A, Sebastián D, Lufrano F, et al. Cost Analysis of Direct Methanol Fuel Cell Stacks for Mass Production. Eng. 2016;9:1008.

[29]	Wang T, Zhao Y, Setzler BP, Abbasi R, Gottesfeld S, Yan Y. A high-performance 75 W direct ammonia fuel cell stack. Cell Rep Phys Sci. 2022;3:100829.

[30]	Cesaro Z, Thatcher J, Bañares-Alcántara R. Chapter 9 - Techno-Economic Aspects of the Use of Ammonia as Energy Vector. In: Valera-Medina A, Banares-Alcantara R, editors. Techno-Economic Challenges of Green Ammonia as an Energy Vector: Academic Press; 2021. p. 209-19.

[31]	Li Y, Taghizadeh-Hesary F. The economic feasibility of green hydrogen and fuel cell electric vehicles for road transport in China. Energy Policy. 2022;160:112703.

[32]	Metzger N, Li X. Technical and Economic Analysis of Fuel Cells for Forklift Applications. ACS Omega. 2022;7:18267-75.

[33]	Afif A, Radenahmad N, Cheok Q, Shams S, Kim JH, Azad AK. Ammonia-fed fuel cells: a comprehensive review. Renew Sustain Energy Rev. 2016;60:822-35.

[34]	Whiston MM, Azevedo IML, Litster S, Samaras C, Whitefoot KS, Whitacre JF. Meeting U.S. Solid Oxide Fuel Cell Targets. Joule. 2019;3:2060-5.

[35]	doosti A, Hoseinpour M, Rasul MG, Karami R, Hassan NMS, Moghtaderi B. A critical review on ammonia as a fuel for internal combustion engines: Is it a viable option? Renew Sustain Energy Rev 2025;222:115964.

[36]	Qi Y, Liu W, Liu S, Wang W, Peng Y, Wang Z. A review on ammonia-hydrogen fueled internal combustion engines. ETransportation 2023;18:100288.

[37]	Brewer EP. On the Use of Carbonate of Ammonia as a Stimulant. Am J Med Sci 1882;84:140–7.

[38]	Zhu G, Tian Y, Liu M, Zhao Y, Wang W, Wang M, et al. Comprehensive competitiveness assessment of ammonia-hydrogen fuel cell electric vehicles and their competitive routes. Energy 2023;285:129471.

[39]	Trangwachirachai K, Rouwenhorst K, Lefferts L, Faria Albanese JA. Recent progress on ammonia cracking technologies for scalable hydrogen production. Curr Opin Green Sustain Chem 2024;49:100945.